# Column Plan

This page is every step needed to figure out your column plan.  Use blank paper for your calculations.

- Record the area of your roof from lesson 1.  Label it Roof Area.

- Record the pounds per square foot for the ground snow load

- The roof snow load is lower than the ground snow load.  Multiply the snow load by 0.7

- Consider the factors on the load adjustment factors handout. Select the appropriate factors for your structure. Multiply the snow load by these factors.  Label this number Modified Snow Load.

- The live load (anything temporarily on the roof, such as people) is 20 psf.  Record the live load.

- Compare the live load and the final snow load.  Choose the number that is larger.  Label this number Roof Loading Factor.

- The dead load (the weight of the roof itself) is 15 psf.  Record the dead load.

- Add the dead load to the roof loading factor. Label this Total Roof Load.

- Divide your Roof Area (recorded above) by 144. Label this Model Area.

- Multiply Total Roof Load by your Model Area. Label this number Total Model Roof Force.

- Divide Total Model Roof Force by the number of columns you want to have in your structure. Label this Force Per Column.

- Use the Balsa Book to determine which kind of wood to use. To use the table, find the part that goes with your ceiling height, and has a maximum safe load that is greater than or equal to your Force Per Column.

- If there is not a strong enough balsa size available, chose a larger number of columns, and recalculate your Force Per Column.

- Draw a footprint of your building on the another page. Draw your columns on the plan where they will go. Label their size.